NETPULSE NOC

Master Reference Architecture, Learning Plan,
and End-to-End Implementation Roadmap

Hybrid Network + IoT + Enterprise Software Platform

Cisco • OSPF • Arduino • Java • Oracle • JavaFX • PHP • MySQL

Prepared by Eng. Mohammed Fares

Architecture Baseline / Engineering Reference

Version 1.0 | 17 August 2026

# Document Control

| Attribute | Value |
| --- | --- |
| Document owner | Eng. Mohammed Fares |
| System | NetPulse NOC — Hybrid Network & IoT Telemetry Monitoring Platform |
| Primary purpose | Single authoritative reference for implementation and future phases |
| Audience | Student developer, network learner, Java developer, DBA, reviewer/CTO |
| Architecture status | Implementation baseline; phase-gated |
| Primary runtime | Java 21 recommended for the main build; Java 17 compatibility considered |
| Desktop | JavaFX |
| Network | Cisco Routers/Switches, OSPF underlay, segmented telemetry path |
| Databases | Oracle for operational telemetry; MySQL for web-application data |
| Web portal | PHP |
| IoT edge | Arduino sensor node |
| Main transport | Cisco -> Middleware: WebSocket/TLS; Arduino -> Middleware: HTTP JSON initially |
| Initial vertical slice | Device Connectivity & Live Status |
| Design principle | Build one complete vertical feature before expanding scope |

How to use this document Do not attempt to implement every section at once. This document is a map. Start at Phase 0, then implement only the current phase acceptance criteria. Treat later phases as future backlog until the current phase is stable.

# 1. Executive Vision

NetPulse NOC is a real-time monitoring platform for a small or medium enterprise network site that combines physical IoT sensing, Cisco networking, enterprise Java processing, Oracle persistence, a JavaFX NOC console, and a PHP/MySQL administration and reporting portal. The system solves a concrete operational problem: an operator often needs to know not only whether a network device is reachable, but also whether the physical environment around that device is healthy, whether routing is stable, whether interfaces are failing, and whether the resulting evidence is recorded and visible to the operations team.

The project is intentionally not defined as “a dashboard”. It is an end-to-end operational system with an edge layer, network transport, event-processing pipeline, durable data store, desktop operations UI, and web administration/reporting surface.

## 1.1 Real-World Problem

Network faults and physical-environment faults are often investigated separately. NetPulse combines them into one operational view.

Polling-only monitoring can delay fault detection. Streaming telemetry reduces detection latency when supported by the device/runtime.

Network connectivity alone is not sufficient evidence of application health. NetPulse tracks device state, telemetry freshness, and processing health.

Operational teams need both a real-time console and a web-based administrative/reporting surface.

The database must preserve enough history to support incident investigation and trend analysis.

## 1.2 The Core Business Scenario

The reference scenario is a “Smart Network Cabinet / Branch Edge Monitoring” deployment. A Cisco router and switch provide network connectivity and dynamic routing using OSPF. An Arduino-based sensor node monitors temperature, humidity, cabinet state, and optionally controls a fan or indicator. NetPulse receives Cisco telemetry and IoT telemetry, correlates the data, raises alerts, persists the operational history, and presents the information to both a JavaFX NOC operator and a PHP/MySQL web portal.

Physical / Network Event
        |
        v
Arduino sensor or Cisco device
        |
        +----------------------+
        |                      |
   HTTP JSON              WebSocket
        |                      |
        +----------+-----------+
                   v
           Java Middleware
                   |
          validate / normalize
                   |
        +----------+----------+
        |                     |
   Oracle persistence      Live event bus
        |                     |
        v                     v
   Historical truth       JavaFX NOC
                              |
                         Operator action
                              |
                              v
                         PHP Web Portal
                              |
                           MySQL

# 2. Project Manifest

## 2.1 Project Manifest — What You Are Building

| Layer | Technology | Role | Must be complete by |
| --- | --- | --- | --- |
| Physical Edge | Arduino + sensor(s) | Generate real physical telemetry and optional actuator response | Phase 2 |
| Network | Cisco Router/Switch + OSPF | Provide routed connectivity and a realistic network underlay | Phase 2 |
| Telemetry Edge | Cisco embedded runtime | Produce JSON telemetry from supported Cisco platform | Phase 6 |
| Core Backend | Java 21 | Receive, validate, normalize, process, persist, and expose APIs | Phase 4 |
| Operational Database | Oracle | Durable network/telemetry/alert system of record | Phase 4 |
| Live Desktop | JavaFX | Real-time NOC operational console | Phase 5 |
| Web Portal | PHP | Administration, reporting, and web workflows | Phase 7 |
| Web Database | MySQL 8.4+ | Users, roles, portal settings, reports, web audit | Phase 7 |
| Architecture | C4 Model + ADRs | Keep the system understandable and traceable | Throughout |
| Testing | JUnit / integration / load | Prove behavior before scale-up | Every phase |

## 2.2 What the Project Is NOT

Not a full Cisco configuration management system.

Not a packet-capture or deep-packet-inspection tool.

Not a microservice laboratory. Start as a modular monolith in Java.

Not a replacement for the enterprise NMS product of a large organization.

Not an automation engine that changes production Cisco configuration in the first release.

Not a database-to-database copy of the same telemetry in Oracle and MySQL.

# 3. Critical Architecture Decisions and Corrections

Decision D-01 — WebSocket server capability JDK java.net.http.WebSocket is a client API. Therefore, the middleware cannot rely on that API alone to accept inbound Cisco WebSocket sessions. Use a server-capable WebSocket runtime/gateway for the inbound endpoint, while the native JDK WebSocket API may still be used for outbound WebSocket clients where appropriate.

Decision D-02 — OSPF is the underlay, not the NetPulse application protocol OSPF provides routing reachability between network segments. NetPulse does not replace OSPF and should not depend on OSPF internals for telemetry processing. NetPulse consumes device/application telemetry over its own application transport.

Decision D-03 — Arduino uses HTTP JSON in the beginner-first implementation Arduino sends a small HTTP JSON payload to a dedicated middleware endpoint. This avoids unnecessary embedded WebSocket complexity while still producing a real networked IoT flow. A WebSocket-based Arduino channel can be added later if the board/library supports it reliably.

Decision D-04 — Oracle and MySQL have different responsibilities Oracle is the operational telemetry and alert system of record. MySQL is the PHP portal application database. Do not duplicate high-volume telemetry into MySQL.

Decision D-05 — Modular monolith first One Java application with strong internal modules is the recommended first implementation. Split services only after measured scaling or failure-isolation requirements justify the operational cost.

## 3.1 Why these decisions matter to a beginner

A common beginner mistake is to interpret “enterprise architecture” as “many technologies and many services”. The opposite is usually more useful: a small number of stable boundaries with clear responsibilities. The NetPulse implementation should therefore grow vertically, not diagonally. Each phase must produce a working system that can be demonstrated, tested, and understood before another technology is introduced.

# 4. Target Architecture — End-to-End

## 4.1 High-Level Logical Architecture

+--------------------------------+
                          |        NOC / Administrators     |
                          +---------------+----------------+
                                          |
                       +------------------+------------------+
                       |                                     |
                 JavaFX Desktop                         PHP Web Portal
                 (real-time NOC)                        (admin/reporting)
                       |                                     |
                 WSS / HTTPS                            HTTPS / HTTP
                       |                                     |
                       +------------------+------------------+
                                          |
                                JAVA MIDDLEWARE
          +--------------------------------------------------------------+
          | Auth/API | WebSocket Ingress | Arduino Ingress             |
          | Decoder  | Event Pipeline     | State Store                 |
          | Alert Engine | Persistence     | Health/Observability       |
          +----------------------+----------------------+--------------+
                                 |                      |
                               JDBC                 MySQL API usage
                                 |                      |
                         +-------+------+       +------+-------+
                         |    Oracle    |       |    MySQL    |
                         | Operational  |       | Web Portal  |
                         |   Truth      |       |    Data     |
                         +--------------+       +-------------+

Network / Physical side:

     Cisco Router ---- OSPF ---- Cisco Router/Switch ---- Arduino Sensor Node
           |                                |                     |
     Cisco telemetry                    network path        HTTP JSON telemetry
     WebSocket/TLS                                            to middleware

## 4.2 Data Ownership Rules

| Data | Authoritative owner | Readers | Writes |
| --- | --- | --- | --- |
| Device inventory | Oracle | Java middleware, JavaFX, PHP through API | Middleware / controlled admin API |
| Interface inventory | Oracle | Middleware, JavaFX, PHP reports | Middleware / controlled admin API |
| Telemetry metrics | Oracle | Middleware, JavaFX queries, PHP reports via API | Middleware only |
| Alert lifecycle | Oracle + Alert Engine | Middleware, JavaFX, PHP reports | Alert Engine + approved operator actions |
| Current runtime state | Java middleware state store | Middleware, live UI publishers | Middleware |
| Web users / roles | MySQL | PHP | PHP administration |
| Web preferences/reports | MySQL | PHP | PHP |
| Routing state | Cisco devices / network | Network engineers; optionally summarized into telemetry | Cisco/network tooling |

## 4.3 System Boundaries

Cisco boundary: devices produce telemetry and participate in the routed network.

IoT boundary: Arduino produces physical measurements and receives only approved control commands.

Middleware boundary: Java is the trust and business-logic boundary.

Oracle boundary: durable operational records.

JavaFX boundary: operator visualization and explicit approved actions.

PHP/MySQL boundary: web administration, reporting, user and portal state.

# 5. Network Design and OSPF Foundation

## 5.1 Lab Topology

OSPF Area 0
          10.10.0.0/30

       +-------------+                  +-------------+
       |  Cisco R1   |------------------|  Cisco R2   |
       | 10.10.0.1   |                  | 10.10.0.2   |
       +------+------+                  +------+------+
              |                                  |
      VLAN 10 Mgmt/IoT                       VLAN 20 Server
      10.10.10.0/24                          10.10.20.0/24
              |                                  |
       +------+-------+                    +-----+----------------+
       | Cisco Switch |                    | NetPulse Host       |
       +------+-------+                    | Java + Oracle       |
              |                            | PHP + MySQL         |
        +-----+------+                     +---------------------+
        | Arduino   |
        | 10.10.10.50
        +-----------+

## 5.2 Roles of the Devices

| Device | Role | Do not use it for |
| --- | --- | --- |
| R1 | OSPF router / edge gateway for the sensor LAN | Application logic |
| R2 | OSPF router / server-side gateway | Database storage |
| Cisco Switch | Layer-2 connectivity; optional VLAN separation | Telemetry processing |
| Arduino | Physical telemetry and actuator | Routing / OSPF |
| NetPulse Host | Application and database services for lab | Enterprise edge routing |

## 5.3 OSPF Learning Objective

For this project, OSPF is useful because it gives you a real reason to study dynamic routing. R1 and R2 exchange routes through OSPF Area 0, so the server network and sensor network remain reachable even if the addressing changes or a secondary routed path is later introduced. The NetPulse application does not “use OSPF” directly; the IP path established by OSPF simply carries application traffic.

## 5.4 Minimal OSPF Lab Configuration Pattern

! R1
router ospf 1
 router-id 1.1.1.1
 network 10.10.0.0 0.0.0.3 area 0
 network 10.10.10.0 0.0.0.255 area 0
 passive-interface GigabitEthernet0/1

! R2
router ospf 1
 router-id 2.2.2.2
 network 10.10.0.0 0.0.0.3 area 0
 network 10.10.20.0 0.0.0.255 area 0
 passive-interface GigabitEthernet0/1

! Verification
show ip ospf neighbor
show ip route ospf
show ip protocols

Exact interface names and supported commands depend on the Cisco platform and IOS/IOS XE release. Use the platform-specific Cisco OSPF configuration guide for the device actually used in the lab.

## 5.5 Network Segmentation — Growth Path

| Segment | Purpose | Initial lab | Future enterprise |
| --- | --- | --- | --- |
| MGMT | Device administration | 10.10.10.0/24 shared for lab | Dedicated management VRF/VLAN |
| IOT/TELEMETRY | Arduino/Cisco telemetry path | Optional separate VLAN | Dedicated telemetry VRF with ACLs |
| APP | Java/PHP services | 10.10.20.0/24 | Application network behind firewall/LB |
| DB | Oracle/MySQL | Same host for lab only | Separate database zone |
| NOC-CLIENT | Operator desktops | Lab PC subnet | Dedicated operator access zone |

# 6. The First Complete Feature — Vertical Slice #1

The first success criterion A single device must appear as ONLINE in JavaFX, be represented in Oracle, be reachable over the Cisco lab network, and be driven by a real telemetry/heartbeat message. If this does not work end-to-end, do not build alerts, topology, historical analytics, or HA.

## 6.1 Feature: Device Connectivity & Live Status

The smallest production-shaped feature is a device heartbeat. The device sends a small message every few seconds. The middleware authenticates or identifies the source, validates the message, updates the device last-seen time and runtime state, writes the durable state to Oracle, and publishes a compact live event to JavaFX. The PHP portal may display the same device status later through the Java API.

{
  "schemaVersion": 1,
  "messageId": "uuid",
  "deviceId": "ARDUINO-LAB-01",
  "timestamp": "2026-08-17T20:00:00.000Z",
  "type": "device.heartbeat",
  "sequence": 1001,
  "payload": {
    "status": "UP"
  }
}

## 6.2 Vertical Slice Acceptance Criteria

| Area | Acceptance criterion |
| --- | --- |
| Network | Arduino can reach the Java host through the Cisco routed path. |
| OSPF | R1/R2 form an OSPF adjacency and both required networks are routable. |
| Java | Middleware accepts one valid heartbeat and rejects malformed payloads. |
| Oracle | Device exists and last_seen_at updates transactionally. |
| API | GET /api/v1/devices returns the device and status. |
| JavaFX | Device row changes from UNKNOWN to ONLINE without UI freeze. |
| Failure | Stopping Arduino eventually produces STALE/DOWN behavior. |
| Recovery | Restarting Arduino returns the device to ONLINE. |
| Observability | Logs expose device ID, connection/session ID, message ID and latency. |

# 7. Arduino / IoT Design

## 7.1 Recommended Hardware Pattern

For a beginner-friendly networked implementation, use an Arduino board that can reach the IP network through Ethernet or Wi-Fi. An Arduino Ethernet Shield Rev2 provides an Ethernet controller and is appropriate when you want to demonstrate a physical wired connection into the Cisco network. An Arduino UNO WiFi Rev2 is another option when your course hardware includes it. The exact board should be selected based on what you physically own.

| Component | Purpose | MVP? |
| --- | --- | --- |
| Arduino board | Execution platform | Yes |
| Temperature sensor (e.g. DHT/BME family) | Physical environment metric | Yes |
| Push button / reed switch | Simulated fault or cabinet door state | Optional |
| LED | Visual local status | Optional |
| Fan / relay | Actuator for over-temperature demo | Phase 8 |
| Ethernet or Wi-Fi module | Network connectivity | Yes, one method required |

## 7.2 Arduino Responsibilities

Read sensors at a controlled interval.

Keep a stable device ID.

Attach a timestamp or monotonically increasing sequence where practical.

Send small JSON payloads.

Retry with bounded backoff after transient network failure.

Never connect directly to Oracle or MySQL.

Never contain the Java middleware business rules.

## 7.3 Arduino HTTP Telemetry Contract

POST /api/v1/iot/telemetry
Content-Type: application/json
Authorization: Bearer <device-token>

{
  "schemaVersion": 1,
  "messageId": "uuid",
  "deviceId": "ARDUINO-LAB-01",
  "timestamp": "2026-08-17T20:00:01.200Z",
  "type": "sensor.metric",
  "sequence": 5001,
  "payload": {
    "sensor": "temperature",
    "value": 28.4,
    "unit": "C"
  }
}

## 7.4 Optional Actuator Path

Java Alert Engine
      |
      | approved control command
      v
POST /api/v1/iot/devices/{id}/commands
      |
      v
Arduino Gateway / device channel
      |
      v
Fan / LED / Buzzer

Keep actuator control out of the MVP. Monitoring first, control second. This avoids turning the first implementation into a safety-critical control system.

# 8. Cisco Telemetry Design

## 8.1 Cisco Role

Cisco devices are network telemetry producers. They should send only to the NetPulse telemetry service endpoint. The Cisco-side design should reference a stable DNS name or VIP rather than a specific middleware node so that a future node replacement does not require device reconfiguration.

Cisco Router / Switch
      |
      | WebSocket + TLS
      | TCP 4242 (if this is the agreed service port)
      v
telemetry.netpulse.example
      |
      +--> Middleware node A
      +--> Middleware node B

Platform-specific caution Cisco embedded JS/IOx capabilities and exact configuration syntax vary by platform and software release. The project reference defines the application behavior and network contract; the exact device CLI must be taken from the documentation for your actual Cisco platform.

## 8.2 Cisco Message Envelope

{
  "schemaVersion": 1,
  "messageId": "9f1d1b6a-4f4e-4a20-a2e4-4ce3d6b6f001",
  "deviceId": "RTR-LAB-01",
  "timestamp": "2026-08-17T20:05:01.120Z",
  "type": "interface.metric",
  "sequence": 184223,
  "payload": {
    "interfaceName": "GigabitEthernet0/0/1",
    "operStatus": "UP",
    "inOctets": 882341223,
    "outOctets": 712321998,
    "inErrors": 2,
    "outErrors": 0,
    "utilizationPercent": 73.4
  }
}

## 8.3 Cisco-to-Middleware Rules

Device identity must be bound to the authenticated session or trusted connection metadata where possible.

Use schemaVersion so the payload can evolve.

Use messageId for idempotency and sequence where available for gap detection.

Use UTC timestamps.

Bound the maximum frame size.

Do not perform Oracle writes inside the WebSocket callback.

Use bounded queues for downstream processing.

Use capped exponential reconnect backoff with jitter.

# 9. Java Middleware — The Core of the System

## 9.1 Recommended Package Structure

netpulse/
  domain/
    device/
    telemetry/
    alert/
    topology/
  application/
    ingestion/
    state/
    alert/
    query/
  infrastructure/
    websocket/
    persistence/
    oracle/
    security/
    observability/
  api/
    rest/
    websocket/
  config/
  bootstrap/

## 9.2 Main Components

| Component | Responsibility |
| --- | --- |
| ConnectionManager | Track device sessions and connection lifecycle. |
| InboundTransport | Accept server-side WebSocket connections from Cisco. |
| ArduinoTelemetryController | Accept HTTP JSON from Arduino. |
| MessageDecoder | Convert transport payload into typed messages. |
| SchemaValidator | Validate required fields and allowed values. |
| Normalizer | Convert device-specific payloads into canonical domain events. |
| EventRouter | Route domain events to state, alert, persistence and live publishing. |
| StateManager | Maintain current device/interface state in memory. |
| AlertEngine | Evaluate operational rules and state transitions. |
| PersistenceQueue | Bound durable write pressure. |
| BatchWriter | Group JDBC writes and commit bounded transactions. |
| DeviceRepository | CRUD and state persistence for devices. |
| MetricsRepository | Batch persistence of telemetry metrics. |
| AlertRepository | Alert lifecycle persistence. |
| QueryService | Read models for the UI and web portal. |
| LiveEventPublisher | Publish filtered, compact events to clients. |
| HealthRegistry | Expose health, queue depth, session count and latency. |

## 9.3 Example Domain Classes

public record DeviceId(String value) {
    public DeviceId {
        if (value == null || value.isBlank()) {
            throw new IllegalArgumentException("deviceId is required");
        }
    }
}

public enum DeviceStatus {
    UNKNOWN, ONLINE, STALE, OFFLINE
}

public record DeviceHeartbeat(
        DeviceId deviceId,
        Instant timestamp,
        long sequence,
        String messageId
) {}

## 9.4 Example Class — DeviceConnectionService

public final class DeviceConnectionService {

    private final DeviceRepository deviceRepository;
    private final LiveEventPublisher eventPublisher;
    private final Clock clock;

    public DeviceConnectionService(
            DeviceRepository deviceRepository,
            LiveEventPublisher eventPublisher,
            Clock clock) {
        this.deviceRepository = deviceRepository;
        this.eventPublisher = eventPublisher;
        this.clock = clock;
    }

    public void heartbeat(DeviceHeartbeat heartbeat) {
        Instant receivedAt = Instant.now(clock);

        DeviceState state = deviceRepository.markOnline(
                heartbeat.deviceId(), receivedAt);

        eventPublisher.publish(
                LiveEvent.deviceStateChanged(state));
    }
}

This class is intentionally small. The controller should decode and validate. The service should coordinate the business operation. The repository should own persistence. The publisher should own live event delivery. Avoid one “God class” that performs networking, parsing, SQL, alerting and JavaFX publishing together.

# 10. Concurrency, Backpressure, and Reliability

## 10.1 The Pipeline

Inbound WebSocket / HTTP
        |
        v
[Bounded Ingress Queue]
        |
        v
[Decode + Validate]
        |
        v
[Normalize]
        |
   +----+----+
   |    |    |
   v    v    v
State Alert Persistence
             |
             v
        [Batch Writer]
             |
          HikariCP
             |
           Oracle

## 10.2 Rules

Never use an unbounded queue in a high-volume path.

Never perform a blocking JDBC operation inside a network callback.

Never allow GUI rendering rate to determine ingestion throughput.

Bound the number of concurrent database operations independently of the number of virtual threads.

Make message processing idempotent where duplicate delivery is possible.

Measure queue depth, age of oldest item, processing latency, active sessions and DB wait time.

Use load shedding or controlled degradation for low-value metrics if the system is persistently overloaded.

## 10.3 Virtual Threads

Java 21 virtual threads can simplify blocking-style application code, especially around JDBC and other I/O-heavy operations. They do not remove the need to bound external resources. Ten thousand virtual threads do not mean ten thousand safe database connections. The Oracle pool is still the hard resource boundary.

## 10.4 Suggested Initial Pooling Values for the Lab

| Resource | Starting value | Why |
| --- | --- | --- |
| Oracle pool max | 10 | Small lab; tune from measurements |
| Oracle pool min idle | 2 | Keep modest warm capacity |
| Batch size | 50–200 rows | Start small and measure commit latency |
| Batch flush interval | 250–1000 ms | Balances freshness and throughput |
| Ingress queue | 1,000–10,000 events | Must be sized from real payloads and memory |
| JavaFX UI refresh | 100–200 ms coalesced | Do not render every telemetry sample |
| Heartbeat | 5–15 s | Choose according to endpoint and network policy |
| Reconnect cap | 60 s | Avoid reconnect storms |

These are starting values for experimentation, not production acceptance values.

# 11. Oracle Data Model and SQL

## 11.1 Core ERD

+--------------------+
| DEVICES            |
|--------------------|
| PK DEVICE_ID       |
| UQ DEVICE_KEY      |
| HOSTNAME           |
| MGMT_IP            |
| DEVICE_TYPE        |
| STATUS             |
| LAST_SEEN_AT       |
+---------+----------+
          | 1
          |
          | N
+---------v----------+
| INTERFACES         |
|--------------------|
| PK INTERFACE_ID    |
| FK DEVICE_ID       |
| INTERFACE_NAME     |
| OPER_STATUS        |
| SPEED_BPS          |
+---------+----------+
          | 1
          |
          | N
+---------v----------+
| METRICS            |
|--------------------|
| PK METRIC_ID       |
| FK DEVICE_ID       |
| FK INTERFACE_ID    |
| EVENT_TIME         |
| METRIC_TYPE        |
| METRIC_VALUE       |
| MESSAGE_ID         |
| SEQUENCE_NO        |
+--------------------+

DEVICES 1 ---- N ALERTS
INTERFACES 1 ---- N ALERTS

## 11.2 MVP Oracle Schema

CREATE TABLE devices (
    device_id        NUMBER GENERATED BY DEFAULT AS IDENTITY,
    device_key       VARCHAR2(100) NOT NULL,
    hostname         VARCHAR2(255) NOT NULL,
    device_type      VARCHAR2(30)  NOT NULL,
    mgmt_ip          VARCHAR2(45),
    site_code        VARCHAR2(50),
    status           VARCHAR2(20) DEFAULT 'UNKNOWN' NOT NULL,
    last_seen_at     TIMESTAMP WITH TIME ZONE,
    created_at       TIMESTAMP WITH TIME ZONE DEFAULT SYSTIMESTAMP NOT NULL,
    updated_at       TIMESTAMP WITH TIME ZONE DEFAULT SYSTIMESTAMP NOT NULL,
    CONSTRAINT pk_devices PRIMARY KEY (device_id),
    CONSTRAINT uq_devices_key UNIQUE (device_key),
    CONSTRAINT ck_devices_status
        CHECK (status IN ('UNKNOWN','ONLINE','STALE','OFFLINE'))
);

CREATE TABLE interfaces (
    interface_id     NUMBER GENERATED BY DEFAULT AS IDENTITY,
    device_id        NUMBER NOT NULL,
    interface_name   VARCHAR2(255) NOT NULL,
    oper_status      VARCHAR2(20),
    speed_bps        NUMBER(20),
    updated_at       TIMESTAMP WITH TIME ZONE DEFAULT SYSTIMESTAMP NOT NULL,
    CONSTRAINT pk_interfaces PRIMARY KEY (interface_id),
    CONSTRAINT fk_interfaces_device FOREIGN KEY (device_id)
        REFERENCES devices(device_id),
    CONSTRAINT uq_interfaces_device_name
        UNIQUE (device_id, interface_name)
);

## 11.3 Metrics and Alerts (Phase 4+)

CREATE TABLE metrics (
    metric_id       NUMBER GENERATED BY DEFAULT AS IDENTITY,
    event_time      TIMESTAMP NOT NULL,
    received_at     TIMESTAMP NOT NULL,
    device_id       NUMBER NOT NULL,
    interface_id    NUMBER,
    metric_type     VARCHAR2(80) NOT NULL,
    metric_value    NUMBER(28,8),
    unit            VARCHAR2(30),
    sequence_no     NUMBER(20),
    message_id      VARCHAR2(100),
    payload_json    CLOB,
    CONSTRAINT pk_metrics PRIMARY KEY(metric_id),
    CONSTRAINT fk_metrics_device FOREIGN KEY(device_id)
        REFERENCES devices(device_id),
    CONSTRAINT fk_metrics_interface FOREIGN KEY(interface_id)
        REFERENCES interfaces(interface_id)
)
PARTITION BY RANGE(event_time)
INTERVAL(NUMTOYMINTERVAL(1,'MONTH'))
(
    PARTITION p_metrics_2026_08
    VALUES LESS THAN (TIMESTAMP '2026-09-01 00:00:00')
);

CREATE TABLE alerts (
    alert_id         NUMBER GENERATED BY DEFAULT AS IDENTITY,
    alert_key        VARCHAR2(200) NOT NULL,
    device_id        NUMBER NOT NULL,
    interface_id     NUMBER,
    severity         VARCHAR2(20) NOT NULL,
    state            VARCHAR2(20) NOT NULL,
    triggered_at     TIMESTAMP NOT NULL,
    acknowledged_at  TIMESTAMP,
    resolved_at      TIMESTAMP,
    summary          VARCHAR2(1000) NOT NULL,
    details_json     CLOB,
    CONSTRAINT pk_alerts PRIMARY KEY(alert_id),
    CONSTRAINT fk_alerts_device FOREIGN KEY(device_id)
        REFERENCES devices(device_id),
    CONSTRAINT fk_alerts_interface FOREIGN KEY(interface_id)
        REFERENCES interfaces(interface_id)
)
PARTITION BY RANGE(triggered_at)
INTERVAL(NUMTOYMINTERVAL(1,'MONTH'))
(
    PARTITION p_alerts_2026_08
    VALUES LESS THAN (TIMESTAMP '2026-09-01 00:00:00')
);

The production schema should be tuned from real query patterns. High-volume telemetry tables should not be overloaded with indexes that are not used by actual dashboard or reporting queries.

# 12. Backend API Contract

## 12.1 API Groups

| Method | Endpoint | Purpose | Phase |
| --- | --- | --- | --- |
| GET | /api/v1/health | Service readiness/liveness | 3 |
| GET | /api/v1/devices | Device inventory | 3 |
| GET | /api/v1/devices/{id} | Device details | 5 |
| POST | /api/v1/iot/telemetry | Arduino telemetry ingest | 2/3 |
| GET | /api/v1/metrics/history | Historical metrics | 6 |
| GET | /api/v1/alerts | Active/historical alerts | 6 |
| POST | /api/v1/alerts/{id}/ack | Acknowledge alert | 6 |
| GET | /api/v1/topology | Topology snapshot | 6 |
| GET | /api/v1/bootstrap | Initial UI snapshot | 5 |
| GET | /api/v1/reports/summary | Web reporting aggregate | 7 |

## 12.2 Live WebSocket Event Contract

{
  "type": "DEVICE_STATE_CHANGED",
  "version": 1,
  "eventId": "uuid",
  "occurredAt": "2026-08-17T20:10:00.500Z",
  "deviceId": "ARDUINO-LAB-01",
  "payload": {
    "status": "ONLINE",
    "lastSeenAt": "2026-08-17T20:10:00.480Z"
  }
}

## 12.3 PHP Integration Rule

PHP should not bypass the Java business layer for operational data. The portal reads network and telemetry information through versioned Java APIs. MySQL remains the PHP application database for users, roles, preferences, reports and web audit data. This prevents the PHP portal from becoming a second, ungoverned network-monitoring backend.

# 13. JavaFX NOC Console

## 13.1 First Screen Only

Do not build the full dashboard initially. Build one screen that lists devices and their state. Once the first row can change in real time from UNKNOWN to ONLINE to OFFLINE, then add charts, topology and alerts.

+-----------------------------------------------------------------------+
| NetPulse NOC        Global Health: HEALTHY      Live: CONNECTED      |
+---------------------+-------------------------------------------------+
| Navigation          | Devices                                         |
|                     |-------------------------------------------------|
| Dashboard           | Device ID       Type      Status   Last Seen   |
| Devices             |-----------------------------------------------|
| Alerts              | ARDUINO-LAB-01  Arduino   ONLINE   2 sec ago  |
| Telemetry           | RTR-LAB-01      Cisco     ONLINE   1 sec ago  |
| Health              | SW-LAB-01       Cisco     OFFLINE  22 sec ago |
+---------------------+-------------------------------------------------+

## 13.2 JavaFX Threading Rule

All network I/O, JSON parsing, database calls and aggregation happen away from the JavaFX Application Thread. Only the final bounded UI delta is applied through Platform.runLater(). OpenJFX explicitly warns against flooding runLater and encourages batching and background work for long-running operations.

// Background thread
stateStore.apply(event);
uiDeltaQueue.offer(toUiDelta(event));

// JavaFX Application Thread
Platform.runLater(() -> {
    var batch = uiDeltaQueue.drain(maxItems);
    applyDeltas(batch);
});

# 14. PHP + MySQL Web Portal

## 14.1 Purpose

The PHP portal provides functions that are naturally web-oriented and do not need the continuous visual refresh behavior of the JavaFX NOC console. Examples include user/role management, reports, saved filters, maintenance windows, and administrative settings.

## 14.2 MySQL Data Model

USERS
  |
  +---- N USER_ROLES N ---- 1 ROLES
  |
  +---- N WEB_AUDIT_LOG
  |
  +---- N REPORTS
  |
  +---- N DASHBOARD_PREFERENCES

## 14.3 Example MySQL Schema

CREATE TABLE users (
    id BIGINT AUTO_INCREMENT PRIMARY KEY,
    username VARCHAR(100) NOT NULL UNIQUE,
    password_hash VARCHAR(255) NOT NULL,
    full_name VARCHAR(200) NOT NULL,
    is_active BOOLEAN NOT NULL DEFAULT TRUE,
    created_at TIMESTAMP NOT NULL DEFAULT CURRENT_TIMESTAMP
) ENGINE=InnoDB;

CREATE TABLE roles (
    id BIGINT AUTO_INCREMENT PRIMARY KEY,
    role_name VARCHAR(50) NOT NULL UNIQUE
) ENGINE=InnoDB;

CREATE TABLE user_roles (
    user_id BIGINT NOT NULL,
    role_id BIGINT NOT NULL,
    PRIMARY KEY(user_id, role_id),
    CONSTRAINT fk_user_roles_user FOREIGN KEY(user_id)
        REFERENCES users(id),
    CONSTRAINT fk_user_roles_role FOREIGN KEY(role_id)
        REFERENCES roles(id)
) ENGINE=InnoDB;

Use InnoDB and explicit foreign-key constraints. MySQL supports transactional behavior and foreign keys for referential integrity; the official reference also documents limitations around partitioned InnoDB tables and foreign keys, which is another reason not to design the web database as a copy of the partitioned Oracle telemetry model.

## 14.4 PHP Data Access Pattern

$pdo = new PDO(
    'mysql:host=127.0.0.1;dbname=netpulse_portal;charset=utf8mb4',
    $username,
    $password,
    [
        PDO::ATTR_ERRMODE => PDO::ERRMODE_EXCEPTION,
        PDO::ATTR_DEFAULT_FETCH_MODE => PDO::FETCH_ASSOC,
    ]
);

PDO_MYSQL is the standard PHP driver for using the PDO interface with MySQL. Use prepared statements and explicit transaction boundaries for multi-step writes.

# 15. C4 Model Reference for the Final System

## 15.1 Level 1 — System Context

NOC Operator ---------> NetPulse NOC <--------- Network Engineer
                             |
                 +-----------+-----------+
                 |                       |
             Cisco Devices           Arduino
                 |                       |
                 +-----------+-----------+
                             |
                         Oracle DB
                             |
                    PHP / MySQL Portal

## 15.2 Level 2 — Containers

+-------------------------------------------------------------------+
|                         NETPULSE NOC                              |
|                                                                   |
|  Cisco Telemetry Ingress     Arduino Telemetry Ingress           |
|                \               /                                  |
|                 v             v                                   |
|                 Java Middleware Service                           |
|                 | State | Alerts | Persistence | APIs | Health   |
|                    |          |         |                           |
|                   Oracle     Live WS   JDBC                      |
|                                |                                  |
|                         JavaFX Desktop                             |
+-------------------------------------------------------------------+

External Web Portal: PHP + MySQL

## 15.3 Level 3 — Components

The main Java container is decomposed into ConnectionManager, WebSocketIngress, ArduinoController, Decoder, Validator, Normalizer, EventRouter, StateManager, AlertEngine, PersistenceQueue, BatchWriter, Repository layer, QueryService, LiveEventPublisher and HealthRegistry.

## 15.4 Level 4 — Code

Represent the code model using packages and focused classes. Keep domain classes framework-light so that they can be unit-tested without a running network or database.

# 16. End-to-End Runtime Flows

## 16.1 Arduino Sensor Flow

Arduino reads temperature
       |
       v
Build JSON envelope
       |
       v
HTTP POST /api/v1/iot/telemetry
       |
       v
Java controller
       |
       v
Validate -> Normalize -> State
       |
       +----> Oracle metric
       |
       +----> Alert Engine
       |
       +----> JavaFX event
       |
       +----> PHP report through API later

## 16.2 Cisco WebSocket Flow

Cisco agent
   |
   | WebSocket/TLS
   v
Server-capable WebSocket endpoint
   |
   v
Session authentication / device identity
   |
   v
Bounded ingress queue
   |
   v
Decode -> validate -> normalize
   |
   +----> State
   +----> Alert Engine
   +----> Persistence
   +----> JavaFX live stream

## 16.3 JavaFX Startup Flow

Login
  -> GET /api/v1/bootstrap
  -> open WSS subscription
  -> buffer live events
  -> apply initial snapshot
  -> replay buffered events
  -> start UI coalescing timer
  -> dashboard operational

## 16.4 PHP Reporting Flow

Browser
  -> PHP session
  -> GET Java /api/v1/reports/summary
  -> Java QueryService
  -> Oracle aggregate query
  -> JSON response
  -> PHP HTML page
  -> optionally store report metadata in MySQL

# 17. Alert Model

## 17.1 Alert Lifecycle

NORMAL
  | threshold crossed
  v
CANDIDATE
  | duration / persistence condition satisfied
  v
ACTIVE
  |
  +--> ACKNOWLEDGED
  |
  | recovery condition
  v
RECOVERED
  |
  v
CLOSED

## 17.2 First Alerts to Implement

| Rule | Trigger | Recovery | Phase |
| --- | --- | --- | --- |
| DEVICE_OFFLINE | No heartbeat for threshold period | Heartbeat received | 6 |
| TEMPERATURE_HIGH | Temperature > 80 C for 3 samples | Temperature < 75 C | 8 |
| INTERFACE_DOWN | Oper status DOWN | Oper status UP | 6 |
| CPU_HIGH | CPU > 90% for sustained period | CPU < 80% | 6 |
| TELEMETRY_STALE | No valid telemetry within threshold | Telemetry resumes | 6 |

Use hysteresis to reduce alert flapping. Use stable deduplication keys such as deviceId + targetId + ruleCode. Persist alert lifecycle transitions rather than only the final state.

# 18. Screen and Portal Roadmap

| Phase | JavaFX | PHP Portal |
| --- | --- | --- |
| 3 | Device list: ONLINE/OFFLINE | Not started |
| 5 | Dashboard + device details | Not started |
| 6 | Alerts + telemetry charts | Not started |
| 7 | Topology + health | Login, users, roles, reports |
| 8 | Historical analytics | Maintenance/report exports |
| 9+ | Advanced NOC workflows | Administration and governance |

# 19. Security Baseline

TLS for operator-to-middleware and device-to-middleware traffic where supported.

Do not store passwords in JavaFX or Arduino source code.

Use device identity and credentials/tokens appropriate to the deployment.

Prefer least-privilege Oracle users: telemetry writer vs query/reporting roles where practical.

PHP users are stored in MySQL with password hashes, never plaintext.

Do not let JavaFX connect to Oracle directly.

Do not let Arduino connect directly to Oracle/MySQL.

Do not expose Oracle or MySQL to the Cisco telemetry network.

Log authentication events and administrative actions with correlation IDs.

Never log access tokens, passwords, database secrets or full sensitive payloads by default.

## 19.1 Suggested Lab Network ACLs

| Source | Destination | Port | Action |
| --- | --- | --- | --- |
| Arduino VLAN | NetPulse App | TCP 8080/8443 (chosen API port) | ALLOW |
| Cisco telemetry | NetPulse telemetry endpoint | TCP 4242 or agreed secure endpoint | ALLOW |
| NetPulse App | Oracle | TCP 1521 or configured Oracle service port | ALLOW |
| PHP/Web | MySQL | TCP 3306 | ALLOW only from portal host |
| Operator desktop | PHP Web | TCP 443 | ALLOW |
| Any edge device | Oracle/MySQL | Any | DENY |

# 20. Testing Strategy

## 20.1 Pyramid

+-------------------+
             | End-to-End Tests  |
             +-------------------+
             | Integration Tests |
             +-------------------+
             | Component Tests   |
             +-------------------+
             | Unit Tests        |
             +-------------------+

## 20.2 Required Tests by Phase

| Phase | Minimum proof |
| --- | --- |
| 1 | Java application starts; configuration loads; tests run. |
| 2 | Arduino sends valid JSON; network path works; invalid input rejected. |
| 3 | Device heartbeat changes Oracle state; API returns device state. |
| 4 | Disconnect/reconnect works; queue remains bounded; no duplicate state. |
| 5 | JavaFX displays live state without UI freeze. |
| 6 | Alert trigger/recovery works; duplicate alerts suppressed. |
| 7 | PHP login/reporting works; MySQL relationships enforce integrity. |
| 8 | Load test demonstrates stable behavior with synthetic concurrent devices. |

## 20.3 Load/Failure Tests

100 simulated device sessions.

250 simulated device sessions before claiming baseline readiness.

Telemetry burst test at increasing event rates.

Artificial Oracle latency injection.

Oracle outage and recovery.

Middleware process restart.

Network path interruption between Cisco and middleware.

JavaFX live event burst with bounded rendering.

24–72 hour soak test before production-style demonstration.

# 21. Observability and Troubleshooting

## 21.1 Metrics to Track

| Metric | Meaning |
| --- | --- |
| active_device_sessions | How many device sessions are currently alive |
| device_reconnect_total | Reconnect frequency |
| ingress_events_total | Total received events |
| ingress_rejected_total | Rejected or malformed events |
| queue_depth | Current backlog |
| queue_oldest_age_ms | How far processing is falling behind |
| batch_commit_latency_ms | Oracle write latency |
| oracle_pool_active | Current connection usage |
| alert_active_total | Current alerts |
| gui_publish_latency_ms | Backend-to-client publish delay |
| api_request_latency_ms | HTTP API latency |

## 21.2 Troubleshooting Decision Tree

Device shows OFFLINE
      |
      +--> ping / routing?
      |       |
      |       +-- No -> check OSPF / gateway / ACL
      |
      +--> API endpoint reachable?
      |       |
      |       +-- No -> check server / firewall / port
      |
      +--> message received in Java logs?
      |       |
      |       +-- No -> inspect auth / payload / endpoint
      |
      +--> Oracle last_seen updated?
      |       |
      |       +-- No -> inspect JDBC / transaction / pool
      |
      +--> JavaFX event received?
              |
              +-- No -> inspect live WebSocket/session

# 22. Repository / Project Structure

netpulse-noc/
├── docs/
│   ├── architecture/
│   ├── network/
│   ├── api/
│   └── runbooks/
├── backend/
│   ├── netpulse-domain/
│   ├── netpulse-application/
│   ├── netpulse-infrastructure/
│   ├── netpulse-api/
│   └── netpulse-bootstrap/
├── desktop/
│   └── netpulse-javafx/
├── web/
│   └── netpulse-portal-php/
├── database/
│   ├── oracle/
│   │   ├── 001_devices.sql
│   │   ├── 002_interfaces.sql
│   │   ├── 003_metrics.sql
│   │   └── 004_alerts.sql
│   └── mysql/
│       ├── 001_users.sql
│       ├── 002_roles.sql
│       └── 003_portal.sql
├── edge/
│   └── arduino-netpulse/
├── network/
│   ├── lab-topology.drawio
│   ├── r1-config.txt
│   └── r2-config.txt
└── tests/
    ├── integration/
    └── load/

# 23. Master Roadmap — Phase by Phase

| Phase | Name | Implementation work | Exit criterion |
| --- | --- | --- | --- |
| Phase 0 | Foundations and environment | Install JDK, Git, IDE, Oracle, MySQL, PHP, JavaFX tooling. Learn IP addressing, TCP, HTTP, JSON, SQL basics. | Everything starts and is versioned. |
| Phase 1 | Hello NetPulse backend | Create Java project, config, health endpoint, logging, Oracle connectivity. | /health returns READY; Oracle connection tested. |
| Phase 2 | Arduino sensor networking | Read one sensor, connect to network, send HTTP JSON. Verify routed path through Cisco. | Real sensor data reaches Java. |
| Phase 3 | Device registry | Create DEVICES table, repository, heartbeat endpoint, API and JavaFX device list. | First complete vertical slice works. |
| Phase 4 | Cisco WebSocket ingestion | Deploy a server-capable WS endpoint, connect one Cisco telemetry producer, normalize events. | Cisco event changes device/interface state. |
| Phase 5 | JavaFX NOC base | Dashboard shell, live stream, coalescing, device details. | Real-time UI works without blocking. |
| Phase 6 | Metrics and alerts | Partitioned metrics, alert engine, lifecycle, acknowledgements. | Operator can detect and investigate a real fault. |
| Phase 7 | PHP/MySQL portal | Users, roles, reports, web audit, admin pages. | Web portal can manage portal concerns and consume operational APIs. |
| Phase 8 | Topology and IoT control | Topology view, Arduino actuator, correlated network/environment scenarios. | Physical fault -> network/application alert -> operator response. |
| Phase 9 | HA and production hardening | Second middleware node, stable endpoint, LB/VIP where appropriate, Oracle HA strategy, observability. | Failure of one application node is recoverable. |
| Phase 10 | Load / chaos / documentation | Synthetic sessions, burst tests, DB outage, long soak, runbooks. | Evidence package supports release readiness. |

## 23.1 Phase Gate Rule

Never move to the next phase because the code compiles. Move only when the current phase has a repeatable demo, automated tests for the new behavior, documented failure behavior, and a short runbook explaining how to reproduce and troubleshoot it.

# 24. Learning Plan — What You Need to Learn

## 24.1 Networking

| Topic | You should be able to do | Priority |
| --- | --- | --- |
| IPv4 / subnetting | Calculate networks, gateways, masks and host ranges | Essential |
| Ethernet / switching | Explain VLANs, ports and basic Layer-2 flow | Essential |
| Routing | Explain next hop, routing tables and default routes | Essential |
| OSPF | Form adjacency, read LSDB-related concepts, verify learned routes | Essential |
| TCP/IP | Explain connection establishment, ports and failure modes | Essential |
| ACLs / firewalling | Create least-privilege connectivity rules | High |
| DNS/NTP | Explain why time and name resolution affect monitoring | High |
| QoS | Understand classes and why telemetry should not starve control traffic | Medium |
| VRF | Understand segmentation concept and route tables | Later |

## 24.2 Java

| Topic | Goal |
| --- | --- |
| Java 17/21 language basics | Records, enums, interfaces, exceptions, generics, collections |
| Concurrency | ExecutorService, CompletableFuture, synchronized/locks, atomics, thread safety |
| Virtual threads | Know where they help and what resource limits remain |
| HTTP/WebSocket | Understand client vs server roles and connection lifecycle |
| JSON | Serialization, schema validation, versioning |
| JDBC | Prepared statements, transactions, ResultSet, batch operations |
| Connection pooling | Understand pool sizing and resource contention |
| Architecture | Layering, ports/adapters, dependency inversion |
| Testing | JUnit, mocks, integration tests, test containers if later adopted |

## 24.3 Oracle

SQL joins, grouping, indexes and execution plans.

Primary keys, foreign keys, unique constraints and check constraints.

Transactions and isolation.

Time-series modeling and partitioning.

Oracle JDBC behavior and connection pooling.

Basic DBA concepts: tablespaces, retention, backup, statistics, AWR/ASH later.

## 24.4 JavaFX

Scene graph, controls, layout, CSS and FXML.

Observable collections and property binding.

Application Thread discipline.

Platform.runLater batching.

Charts and bounded data windows.

ViewModel/Presenter pattern.

## 24.5 PHP/MySQL

PHP 8.x fundamentals and web request lifecycle.

Routing, sessions, authentication and authorization.

PDO/MySQL prepared statements.

Transactions, foreign keys and indexing.

HTML/CSS/JavaScript basics sufficient to build an admin portal.

REST consumption from PHP and secure error handling.

## 24.6 Arduino / IoT

Microcontroller sketch structure and timing.

Digital/analog sensors.

Network client configuration.

HTTP requests and JSON construction.

Retry and timeout behavior.

Basic actuator control and safe defaults.

# 25. Recommended Study Sequence

Learn IPv4 addressing and subnetting until you can design the lab without copying a diagram.

Learn basic Cisco switching and routing, then OSPF Area 0 and verification commands.

Build the Cisco lab and prove end-to-end IP connectivity before touching Java.

Learn Java HTTP and JSON basics; build a tiny HTTP server/client experiment.

Learn Oracle SQL and JDBC; build one repository against DEVICES.

Build the first vertical slice with one Arduino or one simulated device.

Learn Java concurrency and implement bounded event processing.

Build the JavaFX screen after the backend state model is stable.

Add Cisco WebSocket ingestion after the generic event pipeline already works.

Learn PHP sessions/PDO/MySQL and build the portal after operational APIs are stable.

Add alerts, topology, actuator control, HA and load testing only after the core slice is reliable.

# 26. Official Learning Sources and How to Use Them

| Source | URL | Purpose |
| --- | --- | --- |
| Cisco IOS XE OSPF Configuration Guide | https://www.cisco.com/c/en/us/td/docs/switches/lan/c9000/lyr3-fwd/ospf/ospf-configuration-guide/ospf.html | Use for platform-specific OSPF concepts, configuration and verification. The guide was updated July 17, 2026. |
| RFC 2328 — OSPFv2 | https://www.rfc-editor.org/rfc/rfc2328 | Use for protocol-level OSPFv2 understanding and terminology. |
| Java 21 WebSocket API | https://docs.oracle.com/en/java/javase/21/docs/api/java.net.http/java/net/http/WebSocket.html | Use to understand the JDK WebSocket client API and its listener model. |
| Java 21 java.net.http package | https://docs.oracle.com/en/java/javase/21/docs/api/java.net.http/java/net/http/package-summary.html | Use for HttpClient/WebSocket asynchronous behavior and executors. |
| JavaFX Platform.runLater | https://openjfx.io/javadoc/21/javafx.graphics/javafx/application/Platform.html | Use for JavaFX Application Thread rules and batching guidance. |
| Oracle JDBC Developer Guide 21c | https://docs.oracle.com/en/database/oracle/oracle-database/21/jjdbc/index.html | Use for JDBC, connections, statements, transactions and Oracle integration. |
| MySQL 8.4 Reference Manual | https://dev.mysql.com/doc/refman/8.4/en/ | Use for SQL, constraints, transactions, indexes and administration. |
| PHP PDO Manual | https://www.php.net/manual/en/book.pdo.php | Use for database access abstractions and prepared-statement concepts. |
| PHP PDO MySQL | https://www.php.net/manual/en/ref.pdo-mysql.php | Use for PHP-to-MySQL connectivity and driver behavior. |
| Arduino Ethernet Shield Rev2 | https://docs.arduino.cc/hardware/ethernet-shield-rev2/ | Use if the hardware lab uses wired Ethernet; includes Ethernet controller and documentation. |
| Arduino UNO WiFi Rev2 | https://docs.arduino.cc/hardware/uno-wifi-rev2 | Use if you have the Wi-Fi-capable board; review the board and networking resources. |

## 26.1 Learning Rule

Prefer official documentation for protocol and platform facts. Use tutorials and videos for intuition, then return to the official documentation to validate exact commands, API behavior, version compatibility and limitations.

# 27. Engineering Guidelines — Do / Do Not

| Do | Do Not |
| --- | --- |
| Build one end-to-end slice at a time. | Build all screens before integrating the backend. |
| Keep OSPF as the network underlay. | Put routing logic inside the Java business layer. |
| Use Oracle as operational telemetry truth. | Duplicate raw telemetry into MySQL. |
| Use PHP for portal concerns. | Let PHP directly bypass Java for operational writes. |
| Use bounded queues. | Use unbounded queues “because memory is large”. |
| Use server-capable WebSocket ingress. | Assume JDK WebSocket client API can accept inbound sessions. |
| Keep JavaFX updates coalesced. | Call runLater once per telemetry event. |
| Use explicit domain models. | Pass generic Map<String,Object> through the whole system. |
| Measure before tuning. | Choose thread/pool values from intuition alone. |
| Document decisions with ADRs. | Change architecture silently during implementation. |

# 28. Definition of Done for the Full Project

A real Cisco lab path exists and is documented.

OSPF is configured and verified in the lab network.

At least one real Arduino sensor publishes telemetry.

At least one supported Cisco telemetry path is demonstrated.

Java middleware validates, normalizes, processes and persists events.

Oracle stores durable devices, interfaces, metrics and alerts.

JavaFX shows real-time state and alerts without blocking.

PHP/MySQL portal supports user/role administration and reports.

Failure/recovery scenarios are demonstrated.

Load and soak evidence exists for the target lab scale.

Security basics are documented and enforced.

Runbooks exist for the most common failures.

Architecture diagrams match the implemented code and network.

The project can be rebuilt by another developer from the repository and this document.

# 29. Final Implementation Sequence — Your Exact Starting Order

1. Set up Git repository and project folders.

2. Build the Cisco two-router lab and verify OSPF adjacency.

3. Verify end-to-end ping between the sensor network and the server network.

4. Install Java 21 and create the backend project.

5. Create Oracle DEVICES table and a JDBC repository.

6. Create GET /api/v1/health and GET /api/v1/devices.

7. Connect one Arduino to the network and send a heartbeat using HTTP JSON.

8. Implement /api/v1/iot/telemetry.

9. Persist last-seen and device status in Oracle.

10. Build the first JavaFX device list.

11. Add live WebSocket delivery to JavaFX.

12. Break the Arduino connection and verify STALE/OFFLINE behavior.

13. Restore it and verify ONLINE recovery.

14. Tag the repository: vertical-slice-01-device-status.

15. Only after the tag is stable, begin Cisco WebSocket ingestion.

16. Then add metrics, alerts, PHP/MySQL, topology, actuator control, HA and load testing in order.

Most important advice When you feel the project is becoming too large, do not add a new architecture. Reduce the current phase until one small scenario works completely. Your definition of progress is not “I wrote more code”; it is “one operational scenario works from physical/network input to persistence and user-visible output.”

# 30. Existing Project Documents — Consolidation Map

| Previous document | How it is used now |
| --- | --- |
| Technical Project Specification | Global architecture, NFRs, Oracle schema, performance and production principles |
| Cisco Network Design Requirements | Network segmentation, OSPF/routing context, telemetry endpoint and ACL concepts |
| UI Design and Integration Specification | JavaFX screen behavior, API/channel split and rendering rules |
| C4 Model Implementation Guide | Context/container/component/code views and implementation sequencing |
| Master Implementation Plan | Vertical-slice approach and beginner-first phase ordering |

This document supersedes the previous documents as the day-to-day implementation map. The older documents remain useful as detailed subsystem references, but this file decides the order in which you implement them.

# 31. Architecture Sign-Off

| Decision area | Status |
| --- | --- |
| Real-world problem defined | APPROVED |
| Physical IoT integration path defined | APPROVED |
| Cisco/OSPF network role defined | APPROVED |
| Java middleware role defined | APPROVED |
| Oracle/MySQL separation defined | APPROVED |
| JavaFX/PHP responsibilities defined | APPROVED |
| First vertical slice defined | APPROVED |
| Phase gates defined | APPROVED |
| Learning roadmap defined | APPROVED |
| Production hardening deferred until core flow works | APPROVED |