📄 Standard Operating Procedure (SOP): Enterprise Cisco Switch Baseline & Security Hardening

Document Control

- Subject: Global configuration, local authentication, and secure remote access baselines for Cisco IOS networking devices.

- Environment: Production Network Infrastructure.

- Classification: Internal Technical Reference / Engineering Guideline.

- Objective: Standardize initial deployments, mitigate unauthorized access vulnerabilities, and establish robust auditing foundations.

1. The Enterprise Baseline Script (Deployment Ready)

This script is fully optimized for copy-paste deployment via terminal emulators (PuTTY, TeraTerm, SecureCRT). The banner delimiter has been set to the universally safe # character to prevent terminal interrupt errors.

Cisco CLI

! ==========================================================

! 1. DEVICE IDENTITY & DNS MANAGEMENT

! ==========================================================

enable

configure terminal

hostname Multi-SW1

no ip domain-lookup

ip domain-name netpulse.com

! ==========================================================

! 2. ACCESS CONTROL & LOCAL AUTHENTICATION

! ==========================================================

! Using 'secret' instead of 'password' for robust SHA hashing

username admin privilege 15 secret Admin@P@ss2026!

enable secret Enable@P@ss2026!

service password-encryption

! ==========================================================

! 3. LEGAL COMPLIANCE & BANNER

! ==========================================================

! Using '#' as the start and end delimiter for the banner text

banner motd #

*********************************************************

*                 UNAUTHORIZED ACCESS PROHIBITED        *

* This system is strictly for authorized personnel.     *

* All activities are logged and strictly monitored.     *

*********************************************************

#

! ==========================================================

! 4. PHYSICAL & REMOTE MANAGEMENT HARDENING

! ==========================================================

line console 0

login local

logging synchronous

exec-timeout 5 0

exit

line vty 0 15

login local

transport input ssh

logging synchronous

exec-timeout 10 0

exit

! ==========================================================

! 5. COMMIT CHANGES

! ==========================================================

do write

2. Interactive Cryptography Setup (Mandatory Post-Script Step)

Due to Cisco IOS architecture, RSA key generation cannot be automated within a standard copy-paste script without causing syntax errors.

Immediate Post-Script Action:

After pasting the script above, you must manually generate the RSA keys to activate the SSH service:

Cisco CLI

Multi-SW1(config)# crypto key generate rsa

- Prompt: How many bits in the modulus [512]:

- Input: Type 2048 and press Enter.

- Rationale: 2048-bit keys provide military-grade encryption required by modern compliance frameworks (PCI-DSS, HIPAA), whereas 1024-bit is considered weak for modern production environments.

3. Detailed Command Reference, Alternatives & Use Cases

A. Hostname & DNS Resolution

| Command | no ip domain-lookup |
| --- | --- |
| Purpose | Disables broadcast DNS queries when a typo is entered in the CLI. |
| When to Use | On 95% of switches acting as access or distribution layer devices to prevent frustrating CLI freezes (e.g., typing sniping instead of ping). |
| When NOT to Use | Do not use if your switch needs to dynamically resolve external hostnames (e.g., sending logs to syslog.netpulse.com instead of an IP address). |
| The Alternative | If you must have DNS enabled, configure a valid DNS server using: ip name-server 8.8.8.8 to ensure typos resolve instantly. |

B. Privilege Escalation Passwords

| Concept | enable password vs. enable secret |
| --- | --- |
| Legacy Command | enable password 111 stores the password in plain text. Even with service password-encryption (Type 7), it can be easily decrypted using online tools. |
| When to Use Legacy | NEVER in a real-world scenario. Only use it in lab simulations if explicitly requested. |
| Enterprise Standard | enable secret Enable@P@ss2026! uses robust, one-way hashing algorithms (MD5/SHA-256). |
| When to Use Standard | ALWAYS. It acts as the ultimate gatekeeper to Privileged EXEC mode. |

C. Local vs. Centralized Authentication (AAA)

| Concept | Local Database vs. RADIUS/TACACS+ |
| --- | --- |
| Local Config | username admin privilege 15 secret ... combined with login local under lines. |
| When to Use Local | Small networks (1-5 switches), or as a "Fallback / Backdoor" mechanism if the central authentication server goes offline. |
| The Alternative (AAA) | Enterprise networks use TACACS+ or RADIUS (Cisco ISE / Windows NPS). Instead of creating users on 50 separate switches, you manage users in a centralized Active Directory. |

D. Remote Access Hardening & Quality of Life

These commands distinguish a basic configuration from a senior network engineer's baseline:

- transport input ssh (Under VTY Lines):

  - Purpose: Forces all remote sessions to use encrypted SSH, implicitly denying Telnet.

  - When to use: ALWAYS. Telnet sends passwords in clear text; anyone running Wireshark on the network can steal your credentials.

- logging synchronous (Under Console and VTY Lines):

  - Purpose: Prevents annoying asynchronous syslog messages (e.g., "Interface state changed to up") from interrupting the command you are currently typing.

  - When to use: Always. It keeps the CLI clean and prevents typing errors.

- exec-timeout 10 0 (Under Console and VTY Lines):

  - Purpose: Automatically logs the user out after 10 minutes and 0 seconds of keyboard inactivity.

  - When to use: Always. It prevents an administrator from leaving their desk and exposing an open, privileged terminal to unauthorized access.

4. Quality Assurance & Verification (QA Auditing)

After deploying the configuration, execute the following commands in Privileged EXEC Mode (#) to audit system compliance:

| Verification Command | Expected Output & Purpose |
| --- | --- |
| show ip ssh | Verifies SSH is enabled. Look for SSH Enabled - version 1.99 or 2.0. (Use ip ssh version 2 in global config to force modern encryption if needed). |
| `show running-config | section line vty` |
| show users | Displays currently logged-in users. Verifies that remote connections are established via VTY using the correct protocol and local username. |
| show crypto key mypubkey rsa | Validates that the RSA keys have been generated and confirms their modulus size is correct (e.g., 2048 bits). |

📄 Standard Operating Procedure (SOP): Enterprise Cisco Switch Baseline, Security Hardening & VLAN Batch Deployment

Document Control

- Subject: Global configuration, local authentication, secure remote access baselines, and structured VLAN provisioning for Cisco IOS switches.

- Environment: Production Network Infrastructure.

- Classification: Internal Technical Reference / Engineering Guideline.

- Objective: Standardize initial deployments, enforce security best practices, and provision structural segmentation for enterprise departments.

1. The Comprehensive Enterprise Baseline Script (Deployment Ready)

This script integrates the core baseline, security hardening, and a clean batch deployment for departmental and infrastructure VLANs. It is fully optimized for copy-paste deployment via terminal emulators (PuTTY, SecureCRT).

Cisco CLI

! ==========================================================

! 1. DEVICE IDENTITY & DNS MANAGEMENT

! ==========================================================

enable

configure terminal

hostname Multi-SW1

no ip domain-lookup

ip domain-name netpulse.com

! ==========================================================

! 2. ACCESS CONTROL & LOCAL AUTHENTICATION

! ==========================================================

username admin privilege 15 secret Admin@P@ss2026!

enable secret Enable@P@ss2026!

service password-encryption

! ==========================================================

! 3. VLAN ARCHITECTURE & BATCH PROVISIONING

! ==========================================================

interface range gig1/0/4-9

switchport mode trunk

exit

! Setting VTP to transparent prevents accidental VLAN database overwrites

vtp mode transparent

vlan 10

name Sales

!

vlan 20

name HR

!

vlan 30

name Finance

!

vlan 40

name Admin

!

vlan 50

name ICT

!

vlan 60

name SVR

exit

! ==========================================================

! 4. LEGAL COMPLIANCE & BANNER

! ==========================================================

banner motd #

*********************************************************

*                 UNAUTHORIZED ACCESS PROHIBITED        *

* This system is strictly for authorized personnel.     *

* All activities are logged and strictly monitored.     *

*********************************************************

#

! ==========================================================

! 5. PHYSICAL & REMOTE MANAGEMENT HARDENING

! ==========================================================

line console 0

login local

logging synchronous

exec-timeout 5 0

exit

line vty 0 15

login local

transport input ssh

logging synchronous

exec-timeout 10 0

exit

! ==========================================================

! 6. COMMIT CHANGES

! ==========================================================

do write

2. Interactive Cryptography Setup (Mandatory Post-Script Step)

After pasting the script above, execute the following command manually to generate the RSA keys and activate secure SSH communication:

Cisco CLI

Multi-SW1(config)# crypto key generate rsa

- Prompt: How many bits in the modulus [512]:

- Input: Type 2048 and press Enter.

3. Architectural Rationale: VLAN Batch Provisioning

A. VLAN Breakdown and Purpose

| VLAN ID | VLAN Name | Architectural Function & Use Case |
| --- | --- | --- |
| VLAN 10 | Sales | Segregates commercial and sales department endpoints, isolating client data from internal administrative traffic. |
| VLAN 20 | HR | Houses Human Resources assets, protecting confidential employee records and sensitive documentation. |
| VLAN 30 | Finance | Isolates financial systems and accounting terminals, restricting access strictly to authorized personnel via ACLs. |
| VLAN 40 | Admin | Dedicated to executive and administrative management workflows. |
| VLAN 50 | ICT | Reserved for IT administration, management interfaces, and network infrastructure tools. |
| VLAN 60 | SVR | Isolates critical data center servers, hypervisors, and core enterprise application hosts. |

B. Best Practices for VLAN Management

- vtp mode transparent:

  - Why use it: By default, Cisco switches run VTP (VLAN Trunking Protocol) in Server mode. If a switch with a higher revision number connects to the network, it can unintentionally overwrite the entire VLAN database across all switches. Setting VTP to Transparent ensures that the switch manages its own VLAN local database independently, providing maximum safety in modern enterprise environments.

- Descriptive Naming:

  - Why use it: Always assign clear, uppercase/PascalCase names (Sales, ServerRoom) rather than relying purely on numbers. This significantly reduces human error during port-to-VLAN assignment (switchport access vlan 10).

4. Quality Assurance & Verification (QA Auditing)

Execute the following verification commands in Privileged EXEC mode (#) to confirm that both security baselines and VLAN configurations are applied correctly:

| Verification Command | Expected Output & Purpose |
| --- | --- |
| show vlan brief | Displays all configured VLANs (10, 20, 30, 40, 50, 60) along with their active names and associated ports, verifying successful batch provisioning. |
| show vtp status | Confirms that the VTP operating mode is set to Transparent and verifies the local VLAN configuration revision number. |
| show ip ssh | Verifies that SSH is enabled and running version 1.99 or 2.0. |
| `show running-config | section line vty` |

1. Why VTP Mode Transparent Instead of Client?

To understand why Transparent is the modern enterprise standard while Client (and Server) carry significant risks, let's look at how Cisco's VTP (VLAN Trunking Protocol) works and why the VTP Revision Number Trap is a nightmare for network engineers.

A. How VTP Modes Behave

- Server Mode: Can create, modify, and delete VLANs. It broadcasts these changes to other switches in the same VTP domain and increments a Revision Number by 1 with every change.

- Client Mode: Cannot create, modify, or delete VLANs locally. It blindly accepts whatever VLAN database is sent by a VTP Server and overwrites its own configuration to match.

- Transparent Mode: Manages its own VLAN database locally. It does not sync with servers or clients, and it does not overwrite its configuration based on incoming VTP updates (though it will transparently pass VTP advertisements through its trunk ports to other switches).

B. The Danger of VTP Client Mode (Why we avoid it)

Imagine you have 50 switches in Client mode, syncing from a Core switch (Server).

- An old switch is pulled out of a lab—a switch that happens to have the same VTP domain name, but an old, high VTP Revision Number (e.g., Revision 50, while your live production network is at Revision 10).

- When you plug that lab switch into the live network, it announces: "Hey, I am the authority, my revision number is 50!"

- All the switches in Client mode will instantly drop their current VLAN database and adopt the lab switch's database, wiping out all production VLANs across the entire company in seconds. This is a classic, catastrophic real-world network outage.

C. Why Transparent Mode is Safer

In Transparent mode, the switch says: "I don't care what revision number you have or what updates you are sending; I only use the VLANs configured manually in my own local running-config."

- It completely immunizes the switch against accidental or malicious global VLAN wipes.

3. When to Use Each VTP Mode

| VTP Mode | When to Use (Real-World Scenarios) | When NOT to Use / Risks |
| --- | --- | --- |
| Transparent | Recommended Default. Use in almost all modern enterprise environments. Gives you total control to create/delete VLANs locally per switch without any risk of accidental global database corruption. | Do not use if you have a massive campus network and genuinely want automated, centralized VLAN synchronization across hundreds of access switches (though modern automation tools like Ansible or Python scripts have largely replaced VTP for this). |
| Client | Rare / Legacy only. Only used in strictly controlled, legacy environments where a centralized administrative team strictly manages a single VTP Server and wants access switches to automatically inherit VLANs. | Avoid in production. High risk of accidental network-wide outages caused by the VTP revision number trap. |
| Server | Core/Distribution only (if VTP is used at all). Used strictly on designated primary management switches to authoritatively push VLAN databases. | Never enable Server mode on Access switches. If misconfigured, it can take down the entire network domain. |

(Note: In many modern high-security networks, engineers disable VTP entirely using vtp mode off or VTP Version 3 with primary server authorization locks, but Transparent remains the safest and most widely adopted legacy-compatible fallback).